EE6320 RF Integrated Circuits

Project: LNA Design

|  | Performance Summary Table |  |  |
| --- | --- | --- | --- |
| Design metric | Measurement | Simulation Result | Requirement |
| Input matching | Worst case S11 in the specified band | -13.35 | < −10dB |
|  | Band over which S11 ≤ −10dB | 4.989GHz – 5.929GHz | 5.17GHz to 5.32GHz |
|  | Band over which S11 ≤ −15dB | - | - |
| Voltage Gain | Minimum Gain in the specified band | 32.83 | ≥ 20dB |
|  | Maximum Gain in the specified band | 34.05 | ≥ 20dB |
|  | Gain flatness in specified band [Max-Min Gain] | 1.22 dB | ≤ 2dB |
|  | 3dB Bandwidth | 273MHz | - |
|  | Load Capacitance [Differential] | 100fF | 100f F |
| Noise Figure | Maximum Noise Figure in the specified band | 1.539 dB | ≤ 3dB |
|  | Minimum Noise Figure in the specified band | 1.52819 | - |
|  | Band over which NF ≤ 3dB | 3.69 GHz – 6.93 GHz | - |
| Linearity | IIP3 Tones used | 5.245G, 5.246G | - |
|  | Input power used for extrapolation | -70dB | - |
|  | Power of Fundamental Tone at output (at chosen input power) | -35.5 | - |
|  | Power of IM3 Tone at output (at chosen input power) | -162.3 | - |
|  | Extrapolated IIP3 | -6.532 | ≥ -10dBm |
| Power | LNA DC power consumption [Excluding Bias] | 742 uW | Minimize |
|  | Bias circuit power consumption (power supplied by i_bias) | 158.9 uW | Minimize |
| Other | Sum of all on-chip inductances | 11.3n | - |
|  | Sum of all off-chip inductances (2 Lg's) | 9.64n | - |
|  | Sum of all resistances [Including bias] | 200K | - |
|  | Sum of all capacitances [Including AC coupling, excluding load] | 200.236p | - |
|  | Simulator Used | Spectre | - |

Name: Pragadeeswaran Kannan

Roll No: EE21B103

# LNA Schematic

| Component Values (one side values) |  |  |
| --- | --- | --- |
| Design Component | Hand Calculated Value | Simulated Value |
| Lg (gate inductance) | 4.82n | 4.82n |
| Ls (source inductance) | 1.52n | 1.52n |
| Ld (drain inductance) | 4.603n | 4.4n |
| Rd* (drain resistance) | 3034.4 | 2966 |
| R_ls* (res. parallel with Ls) | 1007 | 1007 |
| Rg* (res. parallel with Lg) | 3177 | 3177 |
| Fixed Constant Parameters |  |  |
| ● LNA MOS parameters: W = 15u M, L = 0.13u M |  |  |
| ● Current Mirror MOS parameters: W = 15u M, L = 0.13u M |  |  |
| ● I_bias (current): 300u A |  |  |
| ● C_coupling: 100p each terminal |  |  |

* - intrinsic resistance to respective inductors (doesn’t come in net resistance on/off the chip)

Gain (S21) Plot (only for representation)

Max Gain through hand calculation: <fill here>

Max Gain from the simulation: 34.62

-3dB Gain comes out to be: 31.62

From the figure, 3dB bandwidth comes out to be: 273 MHz

S11 Plot (only for representation)

NF Plot (only for representation)

From hand calculations, NF comes out to be: 1.902 dB

From the plot, NF comes out to be: 1.528 dB

Linearity Plot (IIP3 Computation) (only for representation)

Tones used: 5.245G, 5.246G

IIP3 point comes out to be: -6.533 dB

# LNA Testbench

This testbench is used to compute S11, NF, and IIP3 point.

This testbench is used to compute voltage gain (S21).

Hand Calculations done for the Project

Design Procedure

- Made the schematic from the LNA help document

- Learnt about all the different analysis given there

- Approximately calculated some values and simulated using them

- Then got a good idea from them about the different parameters

- Then calculated all those values again rigorously from scratch

- Tweaked some of them slightly to get the required specifications.

- Got the required LNA amplifier.

# Note

All the images put here (except the test benches) are only for representational purposes, i.e., do not use them as references for your spec values. Use this as a template to make the report (if you are making the report in docs); LaTeX reports are also fine, but maintain the structure as given here.

Path of the library:

Home/ee21b103/cadence_project/RFIC_2024